Selfbow design spreadsheet instructions

Revision 07-04-16	David Dewey (d2dewey@comcast.net)

The intention of this selfbow design spread sheet is to enable anyone with basic wood working (and computer) skills to make a successful, safe (i.e. not likely to break) selfbow at their desired weight and draw. To accomplish this, the sheet calculates the dimensions for a bow with a user defined stress distribution. The stress distribution can uniform, or what ever distribution the designer thinks will be advantageous. Wood with uniform properties and straight grain running most of the length of the bow is assumed. The wood properties need to be measured, or otherwise known, and put into the spread sheet in the provided locations. Wood data taken from standard wood handbooks can be used, but it is obviously better to have test data from a sample of the actual piece of wood the bow is to be made from.

For the purpose of calculation the bow has been “cut” in half, and assumed to be symmetric about the center of the handle. The sheet computes a ½ bow, and then duplicates the missing half to get the complete bow. The ½ bow is further divided in to 5 equal sections starting from the bow center. Each section comprises 20% of the length of the bow from center to the tip of the arm. These sections have been given names as follows: 1) handle/fade  ,  2) inner arm  ,  3) mid arm  ,  4) outer arm , 5) tip.

A separate radius of bend is calculated for each section of the bow. The shape of the fully drawn bow is described by 5 short circular arcs joined together in sequence. In reality the curve of a drawn bow is one smooth curve that is not likely to be perfectly circular. Simulating this curve with 5 separate arcs is a compromise that I hope allows the real shape to be closely modeled without being too complex to manage mentally, or computationally. It allows a stiff handle, and stiff tip section to be defined, while still having 3 arcs left to simulate non circular tiller in the working arms of the bow. It also allows elliptical tiller to be modeled by having the individual radii shortening as they get further from the handle center, or circular tiller to be modeled by setting all radii equal.

In fact, the bend radii are input by the user in a slightly indirect manner, for reasons that I hope will be appreciated after one works with the sheet. The sheet makes use of the fact that the bend radius, thickness, and surface strain level in the bow are directly related. The surface strain(s)  in a bent bow arm is equal to 1/2 thickness(T) divided by the bend radius(R)  or, s = 0.5T/R. (Assuming a rectangular cross section for illustration.) In the interest of designing a bow with known strain, the desired surface strain is directly put into the design sheet by the user. The relative thickness of the bow for each section is also input by the user, along with the thickness at the center of the bow. The sheet then calculates the corresponding bend radius for each section.

Non-rectangular cross sections can be specified, as well as a neutral “plane” location that is adjusted for the cross section at each station. The bow width required to give the desired draw weight, is then calculated for each station along the length of the bow. The resulting bow design has the desired draw weight, and the specified surface strain, and thickness at each station, and a draw length that follows from the 5 bend radii. The final step is to adjust the draw to the desired length by adjusting the overall thickness of the bow. This is done by changing the center thickness scale factor (Tc) until the draw matches the desired length. Alternately the whole bow length can be adjusted to get the desired draw.

The bow shape is calculated and displayed so that the designer can see the effect of the selected strain & thickness choices.

The tiller shape is shown for these cases:

(Shape is shown as the neutral plane without thickness)

1) Unstrung shape including set.

2) Braced shape.

3) Full draw shape.

Bow dimensions and strain are both tabulated and shown graphically as:

1) The bow thickness at each station

2) The bow width at each station

3) The surface strain level at each station

4) The amount of set taken at each station (shown as an amount of residual strain.)

Additional information is also calculated that may be of interest to the bow designer. The mass of the bow is calculated, along with the distribution of the mass by section. The energy stored when braced, and at full draw is calculated, along with the difference in these two energies, again for each station. The total of the energy stored by drawing the bow from brace to full draw is calculated. This energy calculation is surprisingly good, and close to what is actually achieved, without the need to compute a full draw curve.

The bow design sheet is broken into several sections. Some sections are to input information into the sheet regarding the materials, specifications, and design of the bow. Other sections are for output, showing the tiller shape, strains, dimensions, stored energy, and mass details of the bow.

The input sections are marked in the diagram below with black “boxes” around the section. In general, the input cells are colored white and are available to enter and change numbers or formulas. The other cells of the sheet are locked to prevent accidental corruption of the “program”. (Locked cells can be changed by unlocking the sheet from the excel tools, protection menu. The sheet password is “bow”.)

Sample screen shot of the design sheet.

The output sections of the sheet are marked by colored “boxes”. The tan colored cells are the main output cells. You will find the bow construction dimensions in these cells, in addition to the mass (g), and stored energy(J) for each segment of the bow. The blue text, with gray background is the predicted set (in 1/mm units), and the average strain values for set (permanent deformation), and full draw strain for both tension, and compression (active deformation). The compression strain is used to generate the amount of set in combination with the wood properties. The graphs show the strain levels, and bow dimensions along the length of the bow. These graphs can be moved to reveal the working cells underneath, and can also be edited. The bow bending graph (looks like a drawn bow pointing down) shows the bend of the neutral axis for three conditions; un-strung, braced, and full draw. The string is only shown at full draw.

In general the sheet is used as follows:

After setting the specifications, the shape & style of the bow is laid out by putting the details in the bow design input section. This input determines the exact dimensions at each station along the length of the bow. The values to be input are:

Relative thickness (Tr)

Cross-section shape factor (k)

Neutral plane location factor (NPf)

Surface % strain (average for each station)

Initial stave curvature ( C(0) )

Most of the design work for a bow is done by adjusting the surface strain, and relative thickness values along the bow, and observing the resulting width profile, and draw length.

Surface strains of values less than the rupture strain are selected, depending on how much safety margin is desired for the bow, and how much set is acceptable bow. Different wood types will need different surface % strain values. These are found from a bend test (two bend test templates are included in tabs of the selfbow design excel file), or from published wood tables. Brittle wood like cedar needs to have a strain that is determined by the safety margin that is desired relative to the breaking strength. I have used 80% of breaking strength as a reasonable value, but this is dependent on how good the bend test data is for the wood. With brittle wood, an inadequate number of repetitions in flexing the wood in the test, can lead to an over estimation of the breaking strength, and result in exploded bows. With wood like oak, or hickory, set will be the limiting factor for the surface strain values. Surface %strain values that give the maximum acceptable amount of set will be the values to put in the % strain row. Usually the surface strain values will want to be uniform over the working part of the bow arms, and optionally have reduced strain at the handle, and near the tips of the bow. Reduced strain at the handle can be anywhere from nothing, to a few percent (as a slight extra safety margin for the grip), to very low strain with a “non-working” handle bow. The strain at the tips can be reduced to maintain the width of the bow greater than the thickness for stability, or further reduced to produce extra stiff tips.

After the desired strain values are selected, the thickness of the bow at each station will be the primary factor that is changed to produce the desired shape & style of bow. These are input as relative thickness values in the bow design input section. The relative thickness values are multiplied by the center thickness value to give the full thickness of the bow at each station. The input is arranged in this way so that the shape of the bow can be determined by adjusting the relative thickness values without worrying about the resulting draw length. When the shape is as desired, the draw can be finally adjusted by adjusting the center thickness value in the specifications section. Adjusting the center thickness changes the thickness of all portions of the bow at the same time but has only a very minor effect on the resulting computed width shape, while allowing the draw length to be adjusted as desired. The sheet automatically computes the width that is needed to maintain the draw weight, and specified strain values.

Other input that effects the bow design.

Cross-section factors.

The cross section shape factor (k) is used in conjunction with the neutral plane factor (NPf) to specify non-rectangular cross sections. A rectangular cross section has a factor (k) of 1/12 or 0.083 in decimal form, and a neutral plane factor (NPf) of zero. An English long bow abiding exactly by the 5/8 rule, with a fully rounded belly has a k factor of 0.059, and an NPf of -13%. This means that the ELB cross section shape is less stiff by the ratio of .059/.083 compared to rectangular, and that the back has 13% less strain, and belly 13% more strain than a rectangular cross section. There is a tab in the excel file giving diagram examples of k, and NPf factors, and one that allows computation of k, and NPf factors.

Example “k” & “NPf” factors

| description | k | NPf |
| --- | --- | --- |
| crowned back to 50%T/flat belly | 0.057 | 18.80% |
| crowned back to 25%T/flat belly | 0.067 | 7.50% |
| crowned back to 20%T/flat belly | 0.07 | 5.90% |
| crowned back to 15%T/flat belly | 0.073 | 4.30% |
| crowned back to 10%T/flat belly | 0.077 | 2.60% |
| rectangular | 0.083 | 0 |
| Circular or true elliptical | 0.049 | 0 |
| 20% belly / 5% back bevel | 0.073 | -3.60% |
| 25% belly / 10% back bevel | 0.069 | -4.40% |
| 5/8rule full round ELB | 0.059 | -13% |

Note that cross-sections can be compressed in the vertical direction without changing the resulting k, and NPf factors.

Set, reflex, and deflex.

The initial curvature values represent the curvature of the stave prior to any bending stress being applied. If a stave has natural, or induced reflex, or deflex it is input in this row. The units are inverse radius mm. That is; if the bend radius of the stave is 800mm then the curvature is 1/800 = 0.00125. This initial curvature is the base line to which curvature induced by set is added to find the un-strung shape. Reflex is represented by negative(-) curvature values. Deflex, set and the draw of the bow are represented by positive (+) curvature.

The sheet comes with the initial stave curvature set to zero, assuming a straight stave. Try putting positive and/or negative numbers in the initial curvature cells (row labeled “C(0)” ) to see the effect graphically in the bow tiller shape diagram (and bow shape chart tab).

One note of caution here is that the sheet makes a straight line between the tips to determine the string length. Recurved shapes can be specified, but the string will bore right thru the bow to connect the tips, and the calculation will not be accurate. The rule is to keep the string contact to the tips only. Consequently the maximum recurve that the sheet accurately handles is a just barely non-contact recurve when the bow is strung.

Comments and questions to: d2dewey@comcast.net